python3 ~/mate_robotics/Nepi_Connect/tools/fastmjpg_viewer.py --port 5600

2. On the control Pi, start the sender. This version sends to both the laptop and NEPI:

cd ~/rov-26 && ./FastMJPG/bin/FastMJPG capture /dev/v4l/by-path/platform-xhci-hcd.1-usb-0:2.1:1.0-video-index0 640 480 1 30 send 192.168.68.22 5600 192.168.68.12 5600 1400 1000000 2 send 192.168.68.22 5610 192.168.68.23 5600 1400 1000000 1



- On the Pi: python3 -m serial.tools.miniterm /dev/ttyACM0 115200. You should see about 4 $TNCTL lines a second, mostly zeros. Quit with Ctrl+].

- On the laptop: cd ~/rov-26 && ~/rov-venv/bin/python topside/modes/main.py.

- On the Pi: cd ~/rov-26 && python3 rpi/modes/main.py.

- At the bench: turn the supply on and wait for the three tones, then one, then one more.

- Press LB once for slow mode, then move the left stick gently up and down.